Object Type:	funcgen

Description:

This is a function generator which produces periodic output with a waveform

set by the mode field.

Author:		M. Wilson, Caltech (4/89)

------------------------------------------------------------------------------

ELEMENT PARAMETERS

DataStructure:	funcgen_type  [in src/device/dev_struct.h]

Size:		96 bytes

Fields:		mode		flag: 0=sine; 1=square; 2=triangle; 3=constant

amplitude	peak-to-peak amplitude

dc_offset	constant to be added to output

phase		phase in degrees

frequency	frequency

output		output waveform

------------------------------------------------------------------------------

SIMULATION PARAMETERS

Function:	FunctionGenerator  [in src/device/funcgen.c]

Classes:	device

Actions:	CHECK  PROCESS  RESET

Messages:	AMPLITUDE amplitude

DC_OFFSET dc_offset

PHASE phase

FREQUENCY frequency

------------------------------------------------------------------------------

Notes:

In a simulation, you would set up messages between the funcgen element

(in our example it is called "funky") and other elements which are to

receive its output.  For example:

addmsg funky funcplot PLOT output *function *red  // funcplot is an xgraph

addmsg funky Exc_channel ACTIVATION output  // Exc_channel is a synchan

Example:	Scripts/examples/sigmoid

See also: